# TaiWei-Pin-3D — Project Report

Face-to-Face 3D IC Physical Design Flow Engine

CODA-Team · Fudan University · github.com/CODA-Team/TaiWei-Pin-3D

## 0. Black-Box View

For platform integrators: everything you need to invoke TaiWei-Pin-3D without reading the source.

### One-Liner

RTL + PDK → run_experiments.py → GDSII + Metrics Report

### Inputs

| Input | Format | Example | Required |
| --- | --- | --- | --- |
| RTL Design | .v files | designs/src/ibex_sv/*.v | Yes |
| PDK (LEF/LIB/SDC) | Platform directory | platforms/asap7_3D/ | Yes |
| Timing Constraints | SDC file | designs/asap7_3D/ibex/constraint.sdc | Yes |
| Design Config | config.mk | designs/asap7_3D/ibex/config.mk | Yes |

### Outputs

| Output | Path (relative) | Format | Purpose |
| --- | --- | --- | --- |
| Layout GDS | results/<tech>/<case>/<variant>/final.gds | GDSII | Chip fabrication |
| Metrics Report | reports/<tech>/<case>/<variant>/openroad_eval.json | JSON | AI/automated analysis |
| Text Summary | logs/<tech>/<case>/<variant>/openroad/final_summary.txt | TXT | Human review |
| Split-Net Report | logs/.../split_net.split.summary.rpt | KV | HBT count analysis |
| Stage Artifacts | results/<tech>/<case>/<variant>/openroad/*.def | DEF | Resume from checkpoint |
| 3D Views | results/<tech>/<case>/<variant>/openroad/*.png | PNG | Visualization |

### Full Invocation

python3 run_experiments.py --flow ord --tech asap7_3D --case ibex

Runtime: ~50 minutes (ibex)

### Minimal Verification

python3 run_experiments.py --flow ord --tech asap7_3D --case gcd

Runtime: ~15 minutes (smaller design)

Why this matters: The platform adapter layer does NOT need to understand the 18-stage pipeline internals. It only needs to know: call this command, wait for completion, collect results from that directory.

## 1. Project Overview

TaiWei-Pin-3D is an end-to-end reproducible physical design flow for face-to-face (F2F) 3D ICs. It leverages the Pin-3D methodology and mature 2D physical design tools (OpenROAD and Cadence Innovus/Genus) to enable high-quality 3D IC implementation. The project allows academic researchers to validate and compare 3D point tools in a full-flow context.

### 1.1 Key Design Philosophy

Rather than treating 3D IC implementation as a completely new backend flow, Pin-3D reuses mature 2D engines and adds explicit 3D abstractions and stage controls that make the flow 3D-aware while keeping the toolchain practical and reproducible.

### 1.2 Core 3D-Specific Strategies

| Strategy | Description |
| --- | --- |
| Unified 2D Abstraction for F2F | HBTs modeled as special vias in an extended metal stack; existing 2D routers handle cross-tier connections without a custom 3D router |
| Tier-Specific Library Views | Every physical cell has tier-local masters (*_bottom, *_upper); COVER views hide the inactive tier from local optimization while preserving logical connectivity |
| Mixed-Fanout Split | Nets with sinks spanning both tiers are identified, a tier-local split buffer isolates one sink cluster, converting mixed-fanout nets into tier-pure fanout nets |
| Tier-by-Tier Optimization | Placement and legalization alternate between upper and bottom tiers with explicit tier policy, row/site rebuilding, and active-tier control |
| Split PDN + Staged CTS | PDN built separately per tier; CTS structured as owner-tree plus receive-side optimization |
| 3D-Specific Observability | Structural cross_tier nets and functional mixed_fanout nets tracked separately as distinct metrics |

## 2. System Architecture

TaiWei-Pin-3D employs a 4-layer modular architecture with file-based interfaces between layers.

### Layer 1: Orchestration & Dispatch

run_experiments.py (CLI entry point), Makefile + env.sh (stage-level targets), experiment_scripts/, run_experiments_remote.py (cluster dispatch), run_ord.sh / run_cds.sh (quick-start scripts)

### Layer 2: Design & Platform Configuration

designs/: 7 designs × 3 technology nodes with per-design config.mk. platforms/: 3D-enabled PDK with tier-specific LEF/LIB, COVER views, HBT cut layer. designs/src/: RTL Verilog sources.

### Layer 3: Flow Execution Scripts

scripts_openroad/ (66 Tcl): synth → floorplan → 3d_floorplan → tier_partition → split_net → place_* → cts → global_route → detail_route → final_report. scripts_cadence/ (51 Tcl + 2 Py): innovus_3d_* stages, handoff_manager, generate_3d_views.

### Layer 4: Runtime & Output

logs/ + run_logs/ (stage logs, final_summary.txt), objects/ + results/ (DEF, Verilog, openroad_eval.json), reports/ (timing, power, DRC, cross-tier stats).

## 3. 3D IC Physical Design Workflow

The flow proceeds through 5 phases: 2D Bootstrap → 3D Construction → Tier-by-Tier Optimization → Routing → Signoff. Each phase writes canonical handoff files enabling stage-by-stage resume, comparison, and analysis.

### Key 3D Metrics

| Metric | Type | Description |
| --- | --- | --- |
| cross_tier_nets | Structural | Nets still spanning both tiers in merged physical view (HBT pressure) |
| mixed_fanout_nets | Functional | Nets whose real sinks span both tiers (split-net optimization target) |
| hb_via_count | Physical | HBTs assigned by router in the merged 3D stack |
| tier_area_balance | Physical | Utilization ratio between upper and bottom tiers |

## 4. SI2-AI Data Model

### 4.1 Entity-Relationship

Experiment 1──N FlowRun 1──1 Metrics
    │                       ├── FinalMetrics
    │                       └── StageMetrics
    1──1 PlatformConfig
    1──N StageHandoff

### 4.2 Entity Definitions (with Field-Level Schema)

### 4.2.1 Experiment

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| experiment_id | UUID | PK, NOT NULL | Unique experiment identifier |
| flow_type | ENUM('ord','cds') | NOT NULL | Open-source or commercial flow |
| tech | VARCHAR(64) | NOT NULL, FK→PlatformConfig | asap7_3D, nangate45_3D, asap7_nangate45_3D |
| case | VARCHAR(64) | NOT NULL, FK→DesignConfig | gcd, ibex, aes, jpeg, ariane133, bp_quad, swerv_wrapper |
| variant | VARCHAR(128) | NOT NULL | Flow variant name for parallel experiment isolation |
| timestamp_start | DATETIME | NOT NULL, ISO8601 | Experiment start time |
| timestamp_end | DATETIME | NULL | Experiment end time (NULL if still running) |
| status | ENUM('running','completed','failed','timeout') | NOT NULL, DEFAULT='running' | Execution status |
| exit_code | INTEGER | NULL | Process exit code |
| hostname | VARCHAR(256) | NULL | Execution host for traceability |
| orfs_commit | CHAR(40) | NULL | ORFS-Research commit hash (reproducibility) |
| openroad_commit | CHAR(40) | NULL | Bundled OpenROAD commit hash |
| schema_version | INTEGER | NOT NULL, DEFAULT=1 | Schema version for migration compatibility |

### 4.2.2 FlowRun

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| run_id | UUID | PK, NOT NULL | Unique run identifier |
| experiment_id | UUID | FK→Experiment, NOT NULL | Parent experiment |
| stage_name | VARCHAR(64) | NOT NULL | 18 stages: synth, floorplan, floorplan_3d, tier_partition, split_net, place_init, place_init_bottom, place_init_upper, place_bottom, place_upper, place_macro_bottom, place_macro_upper, pdn, pdn_bottom, pdn_upper, cts, global_route, detail_route, final_report |
| stage_order | INTEGER | NOT NULL, CHECK > 0 | Execution order (1-18) |
| tier | ENUM('bottom','upper','merged') | NULL | Which tier this stage operates on |
| status | ENUM('running','completed','failed','skipped') | NOT NULL | Stage status |
| elapsed_sec | FLOAT | NULL | Wall-clock execution time |
| exit_code | INTEGER | NULL | Stage exit code |
| log_file | VARCHAR(512) | NULL | Relative path to stage log |
| tcl_script | VARCHAR(256) | NULL | Primary Tcl script executed |

### 4.2.3 Metrics

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| metrics_id | UUID | PK, NOT NULL | Unique metrics identifier |
| run_id | UUID | FK→FlowRun, UNIQUE | One metrics record per flow run |
| source_file | VARCHAR(256) | NOT NULL | openroad_eval.json or final_summary.txt |
| timing_setup_wns | FLOAT | NULL | Worst negative slack (setup, ns) |
| timing_setup_tns | FLOAT | NULL | Total negative slack (setup, ns) |
| timing_hold_wns | FLOAT | NULL | Worst negative slack (hold, ns) |
| timing_hold_tns | FLOAT | NULL | Total negative slack (hold, ns) |
| wire_length | FLOAT | NULL | Total wire length (μm) |
| drc_errors | INTEGER | NULL, CHECK >= 0 | Design rule check violations |
| cell_count | INTEGER | NULL | Total standard cell instances |
| area | FLOAT | NULL | Total design area (μm²) |
| power_total | FLOAT | NULL | Total power (W) |
| cross_tier_nets | INTEGER | NULL | Nets spanning both tiers (structural) |
| mixed_fanout_nets | INTEGER | NULL | Nets with sinks on both tiers (functional) |
| hb_via_count | INTEGER | NULL | HBT count assigned by router |
| tier_area_bottom | FLOAT | NULL | Bottom tier area (μm²) |
| tier_area_upper | FLOAT | NULL | Upper tier area (μm²) |
| tier_util_bottom | FLOAT | NULL | Bottom tier utilization (0-1) |
| tier_util_upper | FLOAT | NULL | Upper tier utilization (0-1) |
| after_split_buffer_instances | INTEGER | NULL | Buffers inserted during split-net |
| raw_metrics_json | JSON | NULL | Complete raw metrics as extracted |

### 4.2.4 PlatformConfig

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| platform_id | UUID | PK, NOT NULL | Unique platform identifier |
| tech | VARCHAR(64) | UNIQUE, NOT NULL | asap7_3D, nangate45_3D, asap7_nangate45_3D |
| stack_type | ENUM('homogeneous','heterogeneous') | NOT NULL | 3D stack classification |
| top_tech | VARCHAR(32) | NOT NULL | Top tier technology node |
| bottom_tech | VARCHAR(32) | NOT NULL | Bottom tier technology node |
| lef_bottom_path | VARCHAR(512) | NOT NULL | Path to bottom-tier technology LEF |
| lef_upper_path | VARCHAR(512) | NOT NULL | Path to upper-tier technology LEF |
| lib_bottom_path | VARCHAR(512) | NOT NULL | Path to bottom-tier Liberty files |
| lib_upper_path | VARCHAR(512) | NOT NULL | Path to upper-tier Liberty files |
| hbt_layer_name | VARCHAR(32) | NOT NULL | HBT cut layer name (e.g., LVTN) |
| metal_stack_bottom | JSON | NULL | Bottom tier metal layer stack definition |
| metal_stack_upper | JSON | NULL | Upper tier metal layer stack definition |
| drc_rules_path | VARCHAR(512) | NULL | Path to DRC rule deck |

### 4.2.5 StageHandoff

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| handoff_id | UUID | PK, NOT NULL | Unique handoff identifier |
| experiment_id | UUID | FK→Experiment, NOT NULL | Parent experiment |
| stage_name | VARCHAR(64) | NOT NULL | Source stage name |
| file_path | VARCHAR(512) | NOT NULL | Relative path to handoff file |
| file_type | ENUM('def','v','odb','sdc','tcl','json','txt','gds') | NOT NULL | File format |
| file_size_bytes | BIGINT | NULL | File size in bytes |
| sha256 | CHAR(64) | NULL | SHA-256 content hash (reproducibility) |
| tier | ENUM('bottom','upper','merged') | NULL | Which tier this file describes |

### 4.2.6 DesignConfig

| Field | Type | Constraints | Description |
| --- | --- | --- | --- |
| design_id | UUID | PK, NOT NULL | Unique design identifier |
| design_name | VARCHAR(64) | UNIQUE, NOT NULL | gcd, ibex, aes, jpeg, ariane133, bp_quad, swerv_wrapper |
| rtl_source_path | VARCHAR(512) | NOT NULL | Path to RTL Verilog source directory |
| top_module | VARCHAR(128) | NOT NULL | Top-level module name |
| clock_period | FLOAT | NULL | Target clock period (ns) |
| die_area | FLOAT | NULL | Die area (μm²) |
| macro_count | INTEGER | NULL, CHECK >= 0 | Number of macros in design |
| config_mk_path | VARCHAR(512) | NOT NULL | Path to config.mk |
| config_params | JSON | NOT NULL | All config.mk parameter key-value pairs |

### 4.3 JSON Schema Example (openroad_eval.json)

{
  "$schema": "https://json-schema.org/draft/2020-12/schema",
  "title": "OpenROAD Evaluation Metrics",
  "type": "object",
  "patternProperties": {
    "^finish__[a-z]+__[a-z_]+$": {
      "type": "object",
      "properties": {
        "value": { "type": ["number", "string"] }
      },
      "required": ["value"]
    }
  },
  "examples": [{
    "finish__timing__setup__ws": { "value": 0.123 },
    "finish__timing__setup__tns": { "value": -45.6 },
    "finish__route__drc_errors": { "value": 0 },
    "finish__route__wire_length": { "value": 123456 },
    "finish__design__area": { "value": 7890.0 },
    "finish__power__total": { "value": 0.012 }
  }]
}

### 4.4 Persistence Formats

JSON: openroad_eval.json using finish__<category>__<metric> naming convention with {value: number} wrapper objects. TXT: final_summary.txt as space-separated key-value pairs, one per line. Stage Logs: <stage_order>_<stage_name>.log in logs/<tech>/<case>/<variant>/openroad/. Stage Handoffs: DEF, ODB, Verilog per stage with file_type enumeration and SHA256 verification.

### 4.5 SI2-AI Alignment

| SI2-AI Requirement | Implementation |
| --- | --- |
| JSON Schema metrics | finish__<category>__<metric> naming with JSON Schema validation (Section 4.3) |
| Experiment traceability | UUID PKs on all entities; per-stage log files with ordered naming |
| Reproducibility | ORFS + OpenROAD commit hashes; SHA256 on all handoff files |
| Cross-experiment compare | final_summary.txt unified KV format; openroad_eval.json structured JSON |
| State persistence | Stage handoff files enable resume from any stage |
| ORM-ready entities | Field-level type definitions with CHECK constraints map to SQLAlchemy/Django ORM |
| Plugin architecture | scripts_openroad/ + scripts_cadence/ as interchangeable flow backends via --flow flag |
| Schema versioning | schema_version field on Experiment entity for migration compatibility |

### 4.6 Planned Improvements

• Database Backend: Replace file-based logging with SQLite/PostgreSQL using the entity schema above. Each FlowRun writes directly to DB; metrics queries replace grep-based extraction.

• Schema Versioning: Add schema_version field to all entities with Alembic migration scripts.

• Streaming Metrics: Add file-watch based streaming of stage metrics during flow execution, replacing post-hoc log parsing.

• Experiment Registry: Centralized registry with search/filter over all experiments (tech, case, date range, status, PPA range).

• Cross-Experiment GP Reuse: Standardize XY CSV format with metadata columns (tech, case, tool version) for cross-design knowledge transfer.

• Handoff Integrity: Add automated SHA256 verification of handoff files between stages to detect silent corruption.

## 5. Supported Platforms

| 3D Stack | Type | Technologies |
| --- | --- | --- |
| ASAP7 — ASAP7 | Homogeneous | Top: ASAP7 7nm / Bottom: ASAP7 7nm |
| NanGate45 — NanGate45 | Homogeneous | Top: NanGate45 45nm / Bottom: NanGate45 45nm |
| ASAP7 — NanGate45 | Heterogeneous | Top: ASAP7 7nm / Bottom: NanGate45 45nm |

| Design | Description | asap7_3D | nangate45_3D | asap7_nangate45_3D |
| --- | --- | --- | --- | --- |
| gcd | Greatest common divisor | ✓ | ✓ | ✓ |
| ibex | RISC-V core (lowRISC) | ✓ | ✓ | ✓ |
| aes | AES encryption core | ✓ | ✓ | ✓ |
| jpeg | JPEG encoder | ✓ | ✓ | ✓ |
| ariane133 | Ariane RISC-V (133 MHz) | ✓ | ✓ | ✓ |
| bp_quad | BlackParrot quad-core | ✓ | ✓ | — |
| swerv_wrapper | SweRV EH1 RISC-V core | ✓ | ✓ | ✓ |

## 6. Quick Start

source env.sh

python3 run_experiments.py --flow ord --tech asap7_3D --case gcd

bash experiment_scripts/gcd.sh

## 7. Parameter Tuning Space (for Agent/BO Optimization)

For Agent optimization plugins: all tunable parameters, their ranges, and what they affect.

### 7.1 Split-Net Weights (Primary Tuning Target)

Cost function: score = w_util × util_penalty + w_hbt × hbt_penalty + w_area × area_penalty

| Parameter | Env Variable | Default | Range | Effect |
| --- | --- | --- | --- | --- |
| Utilization safe threshold | SPLIT_UTIL_SAFE | 0.70 | 0.50–0.95 | Higher → more HBTs, better timing |
| Alpha (split aggressiveness) | SPLIT_ALPHA | 0.50 | 0.00–1.00 | Higher → more aggressive split |
| Utilization penalty weight | SPLIT_W_UTIL | 1.0 | 0.0–10.0 | Sensitivity to tier utilization |
| HBT penalty weight | SPLIT_W_HBT | 1.0 | 0.0–10.0 | Sensitivity to HBT count |
| Area penalty weight | SPLIT_W_AREA | 1.0 | 0.0–10.0 | Sensitivity to buffer area |

### 7.2 Flow Parameters

| Parameter | Mechanism | Default | Range | Effect |
| --- | --- | --- | --- | --- |
| Parallel jobs | --jobs N | 16 | 1–64 | Throughput |
| DRT iterations | config.mk: DETAILED_ROUTE_END_ITERATION | 8 | 3–20 | Route quality vs. runtime |
| Outer placement iterations | config.mk: OUTER_ITERATIONS | 1 | 1–10 | Placement convergence |
| Core utilization | config.mk: CORE_UTILIZATION ?= | 60 | 20–80 | Cell density, affects congestion |
| Target clock period | SDC file | Design-specific | — | Timing closure difficulty |
| Min routing layer | config.mk: MIN_ROUTING_LAYER | M1 (fixed) | M1–M4 | Routing resource (fixed to M1 for 3D) |

### 7.3 Non-Tunable Parameters (Hardcoded)

| Parameter | Location | Notes |
| --- | --- | --- |
| Place density | scripts_openroad/place_*.tcl | Fixed at 0.6 |
| HBT layer name | platforms/<tech>/lef/ | Defined in technology LEF |
| DESIGN_NICKNAME / DESIGN_NAME | config.mk | Per-design constant (e.g., ibex_core) |

## 8. Platform Integration Recommendations

For the main OpenROAD self-evolution platform: where and how to connect TaiWei-Pin-3D.

### 8.1 Recommended Integration Level: Level 1 (Black-Box Invocation)

Treat the entire TaiWei-Pin-3D as a black-box stage. The platform scheduler invokes run_experiments.py directly.

Rationale: Pin-3D already has complete 18-stage internal orchestration; re-splitting for per-stage integration has prohibitive cost. run_experiments.py already provides status monitoring and metrics collection — sufficient for platform needs. Stage-level hooks are unnecessary — the scheduler only cares about 'started' and 'finished'.

### 8.2 Alternative: Level 2 (Per-Stage Access)

If fine-grained per-stage control is needed, invoke individual Makefile targets: make DESIGN_CONFIG=... ord-synth, ord-floorplan, ord-3d-flow, etc. Cost: requires understanding DEF/ODB file dependencies across 18 stages.

### 8.3 Recommended Hook Type

| Hook | Trigger Point | Purpose |
| --- | --- | --- |
| PreProcessPlugin | Before run_experiments.py invocation | Validate inputs, set env vars |
| PostProcessPlugin | After run_experiments.py completes | Collect outputs, parse metrics, update DB |
| MonitorPlugin | During flow execution | Watch log files for progress, detect stalls |

### 8.4 Interface Contract

Input:  { tech, case, flow, work_dir }

Invoke: python3 run_experiments.py --flow {flow} --tech {tech} --case {case}

Output: { openroad_eval.json, final_summary.txt, final.gds, exit_code }

## 9. Personal Understanding

My own understanding of the TaiWei-Pin-3D project.

### 9.1 What Problem Does This Project Solve

3D IC is not simply stacking two 2D chips together. Traditional 2D EDA tools assume all standard cells are on the same plane with metal routing confined to a single die. Face-to-Face 3D stacking introduces three fundamental changes:

• Physical: Upper and lower tiers each have their own metal stack. Inter-tier connections go through Hybrid Bonding Terminals (HBTs) — not ordinary vias, but connections crossing the bonding interface between two dies.

• Logical: A net's fanout may span both upper and lower tiers simultaneously (mixed-fanout). 2D routers cannot handle this — they see only one die at a time.

• Constraints: Power Delivery Networks (PDN) are independent per tier. The clock tree must be split into owner-tree and receive-side optimization.

The core insight of Pin-3D is: don't rewrite 2D tools. Instead, add a 3D abstraction layer on top of them. Model HBTs as 'special vias' in an extended metal stack so existing 2D routers can handle cross-tier connections. Split mixed-fanout nets into tier-pure branches. Use COVER views so each tier's optimization sees only its own cells.

### 9.2 Why Is the Project Structured This Way

The 4-layer architecture reflects separation of concerns:

• Orchestration Layer (run_experiments.py + Makefile): Separates 'what experiment to run' from 'how to run each stage'. run_experiments.py handles scheduling and status monitoring; the Makefile handles execution order of 18 stages internally.

• Configuration Layer (designs/ + platforms/): Design and technology are fully decoupled. The same design (e.g., ibex) runs on all three platforms (asap7_3D, nangate45_3D, asap7_nangate45_3D) by swapping only config.mk and PDK paths.

• Execution Layer (66 ORD + 51 CDS Tcl scripts): Each script corresponds to one stage. Two interchangeable backends via --flow ord|cds.

• 3D Abstraction Layer: The most innovative layer. Tier Policy Engine controls which tier is active. Split-Net Cost Model decides where to place buffers. 3D View Generator produces merged and per-tier visualizations.

### 9.3 Integration with the Main Platform

From the OpenROAD self-evolution platform perspective, TaiWei-Pin-3D should integrate as a black-box stage plugin:

Input: RTL + PDK + config.mk + SDC

Invocation: python3 run_experiments.py --flow ord --tech asap7_3D --case ibex

Output: GDSII + openroad_eval.json + final_summary.txt + 3D Views

The platform scheduler does not need to understand the 18 internal stages — it only needs to invoke the command, wait for completion, and collect results. If finer-grained per-stage control is needed later, each Makefile target can be invoked individually.

### 9.4 What I Learned

• 3D IC physical design is not a simple extension of 2D. Concepts like tier partition, split-net, COVER views, and staged CTS have no equivalent in 2D design.

• Good engineering architecture means layered decoupling with file-based contracts. Each layer only cares about its own inputs and outputs. Layers communicate through files (DEF, ODB, JSON), not direct memory access.

• The open-source EDA toolchain (OpenROAD + Yosys) is already capable of running complete 3D flows. While some stages (e.g., DRC repair) are less mature than commercial tools, they are fully sufficient as a research platform.

• The parameter tuning space is key for downstream Agent optimization. The 5 split-net weight parameters (util_safe, util_alpha, util_weight, hbt_weight, area_weight) directly affect HBT count and timing quality — they are the most valuable optimization targets.

## 10. References

• Pin-3D Methodology: DOC/openroad_3d_cooptimization_strategy.md

• Cadence 3D Flow: DOC/cadence_3d_cooptimization_strategy.md

• Developer Guide: DOC/openroad_developer_guide.md

• Experiment Monitoring: DOC/run_experiments_monitoring.md

• Known Issues: DOC/issue.md

• ORFS-Research: https://github.com/ieee-ceda-datc/ORFS-Research

CODA-Team · Fudan University · github.com/CODA-Team/TaiWei-Pin-3D